About Sensor:

- Make: Stonex

- Model: X200 GO

- Purpose: Mapping system, uses LiDAR, dual cameras, an IMU and GNSS, genetated 3D images/point clouds.

- Output: .fmlidar, .fmimr, .fmnav, .fmraster

- Software: Slam Go Post

Initial R&D:

- Documentation;

  - Binary and non-ASCII,

  - Contain a header section followed by fixed-length binary records,

  - Store GNSS, IMU, and velocity data in little-endian encoding,

  - Typically use 8-byte doubles and 4-byte floats for navigation fields.

Methodology:

- Metadata:

- Size: 1.32 MB

- CRC64: 0x60105f9832b4bfec

- Content: rtk raw

- Binary header + pattern

  - Read the complete .fmnav file in binary mode.

  - Used ASCII and hex visualization to locate readable headers and recurring byte patterns.

  - Repeating magic sequence AA 44 12 1C, frame boundaries!?

- Frame Segmentation:

  - Divided the binary stream into fixed-size 180-byte records (based on consistent alignment).

  - Verified record count (~14,459 frames) and header consistency.

- Header Analysis:

  - Common header string patterns confirmed GNSS data presence ($GNGGA, $GNRMC).

  - Established Stonex-like binary framing with header + structured payload.

- Progressive Format Testing:

  - Systematically applied candidate struct layouts (<Qddd…, <Qiii…, <Qfff…) to decode payloads.

  - Evaluated outputs by numerical sanity (range, variance, timestamp continuity).

- Pattern Convergence:

  - Identified the best-fit schema at offset +12:  <Qiii fff H → timestamp, 3 × int32 (GNSS/position), 3 × float32 (velocities), uint16 (quality).

- Validation:

  - Timestamp deltas ~2.1×10¹¹ ns ⇒ ~4.8 Hz GNSS update rate.

  - Position values consistent in a small meter-scale range (x≈55–416 m, y≈1179 m).

  - Lat/Lon scaled (×1e-7) gave near-zero geodetic drift → local coordinate system confirmed.

Findings

- Header: AA 44 12 1C, repeated every 180 B.

- Payload begins at byte 12 after header.

- Primary GNSS record format: <Qiii fff H (timestamp + pos + vel + quality).

- Timestamp continuity confirms consistent sampling (≈0.2 s interval).

- Remaining ~146 B per frame likely store IMU/attitude data.

- File mixes GNSS (textual NMEA) and binary LiDAR/IMU blocks, typical for Stonex X200 GO.

References:

- https://www.mdpi.com/2072-4292/15/2/374#:~:text=laser%20data%2C%20load%20IMU%20file%2C,is%20shown%20in%20Table%202

- https://stonexdocs.s3.eu-central-1.amazonaws.com/Catalog/STONEX_CATALOG_2025.pdf#:~:text=The%20rotating%20head%20has%2032,high%20precisions%20IMU%20makes%20the

- https://www.qso.com.ar/datasheets/Receptores%20GNSS-GPS/NMEA_Format_v0.1.pdf